# A CAPSTONE PROJECT REPORT

## SMART IRRIGATION DECISION SUPPORT SYSTEM USING SIMULATED SOIL MOISTURE AND WEATHER DATA

A Capstone Project Report submitted in partial fulfilment of the requirements for the course

### CSA 0904 – JAVA PROGRAMMING

to the award of the degree of

## BACHELOR OF TECHNOLOGY

### IN

[COMPUTER SCIENCE AND ENGINEERING (CSE)]

### Submitted by

O. Bhanu Prakash Reddy – 192372075
Mandati Sharan Mahi Reddy – 192324082
Kandath Haneesh – 192324084
P. Sri Hari – 192372222
A Ajay Antony – 192524414

### Under the Supervision of

Dr. Thalapathy Rajasekaran

### SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences
Chennai – 602105

### SEPTEMBER 2026

# SIMATS ENGINEERING

# Saveetha Institute of Medical and Technical Sciences

# Chennai-602105

# BONAFIDE CERTIFICATE

This is to certify that the Capstone Project entitled “Smart Irrigation Decision Support System Using Simulated Soil Moisture and Weather Data” has been carried out by O. Bhanu Prakash Reddy (192372075), M. Mahi Reddy (192324082), K. Haneesh (192324084), Sri Hari (192372222), and Antony Ajay (192524414) under the supervision of Dr. Thalapathy Rajasekaran and is submitted in partial fulfilment of the requirements for the current semester of the B.Tech programme at Saveetha Institute of Medical and Technical Sciences, Chennai.

The project work presented in this report is based on the design and implementation of a Java-based decision support application that simulates soil moisture and weather conditions and provides rule-based irrigation recommendations.

#### COURSE COORDINATOR	COURSE FACULTY

Dr. G. Charlyn Pushpa Latha,	DR. Thalapathy Rajasekaran,

Professor	Professor

Department of CSE	Department CSE

SIMATS Engineering,	SIMATS Engineering,

SIMATS, Chennai – 602105.	                                    SIMATS, Chennai 602105.

# DECLARATION

We, the students of the [COMPUTER SCIENCE AND ENGINEERING (CSE)], SIMATS Engineering, Saveetha Institute of Medical and Technical Sciences, Chennai, hereby declare that the Capstone Project Work entitled “Smart Irrigation Decision Support System Using Simulated Soil Moisture and Weather Data” is the result of our own bonafide efforts.

To the best of our knowledge, the work presented herein is original and has been carried out in accordance with the principles of engineering ethics and academic integrity. All sources, references, data, software resources, and other materials used in the project have been appropriately acknowledged.

The system developed in this project uses simulated soil moisture and weather data for demonstrating the irrigation decision-making process. No claim is made that the simulated values represent measurements obtained from real agricultural sensors.

We further declare that the data, results, analysis, and findings presented in this report have not been deliberately fabricated, falsified, or manipulated. External software tools, computational resources, datasets, and AI-assisted resources used during the project have been appropriately disclosed.

We confirm that all applicable academic and professional requirements have been followed during the planning, development, testing, analysis, and documentation of the project.

Place:Chennai
Date: _____________

Signature of the Students with Names

O. Bhanu Prakash Reddy

M. Mahi Reddy

K. Haneesh

P. Sri Hari

Antony Ajay

INDIVIDUAL CONTRIBUTION STATEMENT

| Student | Specific Responsibilities | Design & Development Contribution | Testing & Analysis Contribution | Report Contribution | Approx. Contribution |
| --- | --- | --- | --- | --- | --- |
| O. Bhanu Prakash Reddy | System planning and Java implementation | Decision-support modules | Functional testing | Chapters 1 & 4 | 20% |
| M. Mahi Reddy | Data simulation and farm management | Simulation module | Test-data analysis | Chapter 2 | 20% |
| K. Haneesh | Irrigation decision logic | Rule-based engine | Decision verification | Chapter 3 | 20% |
| Sri Hari | Recommendation and reporting | Alert/report modules | Output validation | Chapters 4 & 5 | 20% |
| Antony Ajay | Documentation and integration | System integration | Final testing | References & Appendices | 20% |
| Total |  |  |  |  | 100% |

ABSTRACT

Agriculture depends heavily on effective irrigation management, but traditional irrigation decisions are often based on manual observation, fixed schedules, or personal experience. Such methods may result in excessive water usage when irrigation is applied unnecessarily or crop water stress when irrigation is delayed. Smart irrigation technologies can address this challenge by combining environmental information with decision-making algorithms. However, real-world smart irrigation systems generally require sensors, communication infrastructure, controllers, and continuous maintenance, which can increase development cost and complexity.

This project presents a Smart Irrigation Decision Support System Using Simulated Soil Moisture and Weather Data. The proposed system is implemented as a Java-based application and provides a hardware-free environment for demonstrating intelligent irrigation decision-making. Instead of collecting data from physical sensors, the system generates simulated values for soil moisture, soil temperature, air temperature, humidity, rainfall, wind speed, solar radiation, and forecast rainfall.

The system is organized into five major modules: User and Farm Management, Soil Moisture and Weather Data Simulation, Irrigation Decision Engine, Recommendation and Alerts, and Report Generation and History. The decision engine evaluates environmental conditions using predefined rules. For example, irrigation is recommended when soil moisture falls below the minimum threshold, rainfall is absent, and significant rainfall is not expected. The system can also identify critical moisture conditions, delay irrigation when rainfall is expected, and prioritize areas requiring immediate attention.

The project demonstrates the use of Java programming and object-oriented programming principles in solving a practical agricultural problem. The simulation approach allows the complete decision-making workflow to be developed and tested without requiring physical sensors or irrigation hardware.

The proposed system serves as a foundation for future integration with real soil-moisture sensors, weather APIs, IoT devices, databases, and machine-learning models.

Keywords: Smart Irrigation, Java, Soil Moisture, Weather Simulation, Decision Support System, Rule-Based Decision Making

# TABLE OF CONTENTS

| Sl. No. | Title |
| --- | --- |
| i | Certificate |
| ii | Declaration |
| iii | Individual Contribution Statement |
| iv | Abstract |
| v | List of Figures |
| vi | List of Tables |
| vii | List of Abbreviations |
| 1 | Chapter 1 – Introduction and Engineering Problem |
| 2 | Chapter 2 – Literature Review and Existing Solutions |
| 3 | Chapter 3 – Engineering Design & Trade-offs, Sustainability, Ethics, Safety, Risk & Security |
| 4 | Chapter 4 – Implementation, Testing & Results, Project Management |
| 5 | Chapter 5 – Conclusion, Future Work and Reflection |
| 6 | References |
| 7 | Appendices |

# LIST OF FIGURES

| Figure No. | Figure Name |
| --- | --- |
| Figure 3.1 | Overall System Architecture |
| Figure 3.2 | Soil Moisture and Weather Simulation Model |
| Figure 3.3 | Irrigation Decision Engine |
| Figure 3.4 | System Data Flow |
| Figure 3.5 | Decision-Making Flowchart |
| Figure 4.1 | Java Application Interface |
| Figure 4.2 | Simulated Soil Moisture Output |
| Figure 4.3 | Irrigation Recommendation Output |
| Figure 4.4 | Alert Generation Output |
| Figure 4.5 | Irrigation History Report |
| Figure 4.6 | Project Development Timeline |

# LIST OF TABLES

| Table No. | Table Name |
| --- | --- |
| Table 1.1 | Project Objectives |
| Table 2.1 | Existing Irrigation Approaches |
| Table 3.1 | Stakeholder Requirements |
| Table 3.2 | Functional Requirements |
| Table 3.3 | Non-Functional Requirements |
| Table 3.4 | Design Specifications |
| Table 3.5 | Alternative Solution Evaluation |
| Table 3.6 | Trade-off Analysis |
| Table 3.7 | Sustainability and Ethics Analysis |
| Table 3.8 | Risk Register |
| Table 4.1 | Development Resources |
| Table 4.2 | Software Tools |
| Table 4.3 | Test Plan |
| Table 4.4 | Representative Simulation Results |
| Table 4.5 | Objective Achievement |
| Table 4.6 | Strengths and Limitations |
| Table 4.7 | Project Schedule |
| Table 4.8 | Project Budget |

# LIST OF ABBREVIATIONS / SYMBOLS

| Symbol / Abbreviation | Description |
| --- | --- |
| AI | Artificial Intelligence |
| API | Application Programming Interface |
| DSS | Decision Support System |
| IoT | Internet of Things |
| OOP | Object-Oriented Programming |
| Java | Java Programming Language |
| SM | Soil Moisture |
| RH | Relative Humidity |
| RF | Rainfall |
| UI | User Interface |
| DB | Database |
| °C | Degree Celsius |
| % | Percentage |
| mm | Millimetre |
| km/h | Kilometres per hour |
| W/m² | Watts per square metre |

# CHAPTER 1

# INTRODUCTION AND ENGINEERING PROBLEM

# 1.1 Background

Agriculture is one of the major sectors requiring efficient utilization of natural resources. Among the different resources required for crop production, water is particularly important because insufficient irrigation can affect crop growth while excessive irrigation can result in water wastage, soil problems, and unnecessary energy consumption.

Traditional irrigation practices often depend on fixed irrigation schedules or manual observation. A farmer may inspect soil conditions and weather conditions and then decide whether irrigation should be performed. Although this approach can be effective when based on experience, it does not consistently consider multiple environmental factors simultaneously.

A decision support system can assist by combining environmental information and predefined decision rules. Instead of simply asking whether the soil is dry, the system can consider soil moisture, rainfall, temperature, humidity, and expected rainfall before making a recommendation.

The proposed project develops a Java-based Smart Irrigation Decision Support System that simulates soil moisture and weather information. The project does not require physical sensors. Instead, simulated values are generated and processed to demonstrate the complete workflow.

The project material identifies the system as a Java-based application that simulates soil moisture and weather conditions and analyzes those conditions using predefined decision rules.

The system consists of five major functional modules:

- User and Farm Management

- Soil Moisture and Weather Data Simulation

- Irrigation Decision Engine

- Recommendation and Alerts

- Report Generation and History

These modules allow the project to demonstrate how environmental information can be converted into actionable irrigation recommendations.

## 1.2 Need for the Project

Efficient irrigation is important because both under-irrigation and over-irrigation can negatively affect agriculture.

### Problems associated with under-irrigation

- Crop water stress

- Reduced growth

- Reduced yield

- Soil moisture deficiency

- Increased risk during high-temperature periods

### Problems associated with over-irrigation

- Water wastage

- Increased pumping requirements

- Possible soil nutrient loss

- Unnecessary operational cost

- Reduced irrigation efficiency

A major challenge is that small-scale projects and educational demonstrations may not have access to physical sensors and IoT infrastructure. Therefore, a simulation-based decision support system provides a practical method for demonstrating smart irrigation concepts.

The proposed system is useful for:

- Demonstrating agricultural decision-support concepts.

- Understanding rule-based irrigation logic.

- Learning Java programming through a practical application.

- Simulating different environmental conditions.

- Testing irrigation recommendations without hardware.

- Providing a foundation for future IoT integration.

The project objectives specifically include simulating soil moisture, temperature, humidity, rainfall and implementing rule-based irrigation decisions.

## 1.3 Problem Statement

Consider a farmer managing multiple agricultural zones. Every day, the farmer needs to determine whether each zone requires irrigation. A manual decision based only on visual observation may not consider soil moisture, rainfall, temperature, humidity, and future rainfall simultaneously.

The engineering problem is therefore:

To design and implement a Java-based decision support system capable of simulating soil moisture and weather conditions and using these interacting parameters to determine whether irrigation is required, delayed, or unnecessary.

The system must operate under realistic constraints such as:

- No physical IoT sensors.

- No dedicated irrigation controller.

- Limited project cost.

- Simulated rather than real-time environmental data.

- Need for understandable decision rules.

- Requirement for repeatable testing.

- Need to maintain irrigation history.

The project material describes the same underlying problem using a farmer who may over-irrigate on some days and delay irrigation on other days because decisions are based on manual observation and experience.

## 1.4 Project Aim

The main aim of this project is:

To design and develop a Java-based Smart Irrigation Decision Support System that simulates soil moisture and weather data and provides rule-based irrigation recommendations, alerts, and historical reports.

## 1.5 Project Objectives

The following measurable objectives were established:

| No. | Objective |
| --- | --- |
| 1 | To design a Java-based decision support system for irrigation management. |
| 2 | To simulate soil moisture, temperature, humidity, rainfall, and related weather parameters. |
| 3 | To develop a rule-based irrigation decision engine. |
| 4 | To determine irrigation requirements based on soil moisture and weather conditions. |
| 5 | To generate irrigation recommendations and alerts. |
| 6 | To maintain irrigation history and generate reports. |
| 7 | To demonstrate Java OOP concepts through an agricultural application. |

These objectives are consistent with the project's original objective list.

## 1.6 Scope

### Included

The project includes:

- Farmer/user management.

- Farm and zone configuration.

- Crop information.

- Soil moisture threshold configuration.

- Simulated soil moisture.

- Simulated weather information.

- Rule-based irrigation decisions.

- Irrigation recommendations.

- Alert generation.

- Irrigation history.

- Basic reports.

- Java-based implementation.

### Excluded

The following are outside the present project scope:

- Physical soil-moisture sensors.

- Real-time weather stations.

- Automatic water pumps.

- Physical irrigation valves.

- Industrial IoT communication.

- Large-scale agricultural deployment.

- Machine-learning-based prediction.

- Real-time cloud deployment.

### Assumptions

- Simulated values are representative and do not represent physical measurements.

- The user provides appropriate soil-moisture thresholds.

- Weather parameters are generated within meaningful ranges.

- Crop water requirements are represented through simplified rules.

### Boundary Conditions

The system operates within a software simulation environment. Its decisions are based on the parameters supplied by the simulation module and configured thresholds.

## 1.7 Expected Engineering Outcome

The expected outcome is a functional Java-based decision support application capable of:

- Generating environmental data.

- Classifying soil conditions.

- Evaluating rainfall conditions.

- Applying irrigation rules.

- Generating recommendations.

- Producing alerts.

- Maintaining irrigation history.

- Generating summary information.

The system is intended to serve as a prototype decision-support model rather than a direct replacement for a physical irrigation control system.

# CHAPTER 2

# LITERATURE REVIEW AND EXISTING SOLUTIONS

## 2.1 Review Method

The literature review was structured around the major technologies involved in the project:

- Smart irrigation systems.

- Soil-moisture-based irrigation.

- Weather-based irrigation.

- Rule-based decision systems.

- IoT agriculture.

- Simulation-based agricultural applications.

- Java-based decision-support applications.

The review focuses on understanding how environmental parameters can be used to improve irrigation decisions and identifying the limitations of systems requiring physical sensors and IoT infrastructure.

## 2.2 Review of Existing Work

### 2.2.1 Traditional Manual Irrigation

Traditional irrigation commonly depends on farmer experience, visual inspection, crop condition, and fixed schedules.

The primary advantage is simplicity. However, the approach is subjective and may not respond rapidly to changing environmental conditions.

### 2.2.2 Timer-Based Irrigation

Timer-based systems activate irrigation according to a predefined schedule.

Advantages include:

- Simple operation.

- Low implementation complexity.

- Predictable irrigation timing.

However, timer-based systems do not necessarily consider current soil moisture or rainfall.

### 2.2.3 Soil-Moisture-Based Irrigation

Soil-moisture-based systems use soil-moisture information to determine whether irrigation is necessary.

Such systems are more adaptive than fixed timers because irrigation decisions are connected to actual or estimated soil conditions.

However, real systems require:

- Soil-moisture sensors.

- Sensor calibration.

- Data acquisition.

- Hardware maintenance.

### 2.2.4 IoT-Based Smart Irrigation

IoT-based irrigation systems can combine sensors, microcontrollers, communication networks, cloud platforms, and dashboards.

These systems can provide:

- Remote monitoring.

- Automated irrigation.

- Real-time data.

- Historical information.

However, they can increase system complexity and cost.

### 2.2.5 Weather-Based Irrigation

Weather-based irrigation considers parameters such as:

- Temperature.

- Humidity.

- Rainfall.

- Forecast rainfall.

- Solar radiation.

- Wind.

Weather information can help prevent unnecessary irrigation when rainfall is expected.

### 2.2.6 Rule-Based Decision Systems

A rule-based system uses predefined conditions to convert input parameters into decisions.

For example:

IF

Soil Moisture < Minimum Threshold

AND

Rainfall = 0

AND

Rain Forecast = Low

THEN

Irrigation = Required.

This is the primary decision approach used in the proposed project. The project's original material specifies this rule directly.

## 2.3 Comparative Analysis

| Existing Approach | Advantages | Limitations | Relevance |
| --- | --- | --- | --- |
| Manual irrigation | Simple and inexpensive | Subjective, inconsistent | Problem basis |
| Fixed timer | Easy to implement | Ignores soil/weather conditions | Baseline |
| Soil moisture system | Uses soil condition | Requires sensors | Future extension |
| IoT irrigation | Real-time monitoring | Higher cost and complexity | Future extension |
| Weather-based system | Considers rainfall/weather | Depends on weather data | Relevant |
| Proposed simulation | Low cost, repeatable, hardware-free | Uses simulated data | Current project |

## 2.4 Research / Engineering Gap

The major engineering gap addressed by this project is the absence of a simple, low-cost, hardware-independent platform for demonstrating the complete smart irrigation decision-making process.

Real smart irrigation systems can require:

- Sensors.

- Microcontrollers.

- Wireless communication.

- Databases.

- Cloud platforms.

- Physical irrigation equipment.

Such requirements can make development difficult for an academic Java programming project.

The proposed system fills this gap by simulating environmental data and focusing on the decision-making layer.

## 2.5 Proposed Contribution

The main contribution of the project is a Java-based software framework that integrates:

- User management.

- Farm configuration.

- Environmental simulation.

- Rule-based decision-making.

- Irrigation recommendations.

- Alerts.

- Historical records.

- Reports.

The system therefore provides a complete demonstration of how environmental data can be transformed into irrigation decisions without requiring physical hardware.

# CHAPTER 3

# ENGINEERING DESIGN & TRADE-OFFS, SUSTAINABILITY, ETHICS, SAFETY, RISK & SECURITY

## 3.1 Requirements

### 3.1.1 Stakeholder / User Requirements

| Stakeholder | Requirement |
| --- | --- |
| Farmer/User | Easy-to-understand irrigation recommendations |
| Farmer/User | View soil and weather conditions |
| Farmer/User | Configure farm and crop details |
| Farmer/User | Receive low-moisture alerts |
| Farmer/User | View irrigation history |
| Student/Developer | Easy-to-test software architecture |
| Faculty/Evaluator | Demonstration of Java and OOP concepts |

The User and Farm Management module allows users to configure farmer details, farm information, crops, zones, and soil moisture thresholds.

### 3.1.2 Functional Requirements

| ID | Functional Requirement |
| --- | --- |
| FR1 | User registration and login |
| FR2 | Farm and zone management |
| FR3 | Crop configuration |
| FR4 | Soil moisture threshold configuration |
| FR5 | Generate simulated environmental data |
| FR6 | Analyze soil moisture |
| FR7 | Analyze rainfall conditions |
| FR8 | Apply irrigation decision rules |
| FR9 | Generate irrigation recommendations |
| FR10 | Generate alerts |
| FR11 | Store irrigation history |
| FR12 | Generate reports |

### 3.1.3 Non-Functional Requirements

| Requirement | Target |
| --- | --- |
| Usability | Simple interface and understandable output |
| Performance | Fast decision generation for simulated input |
| Reliability | Consistent rule execution |
| Maintainability | Modular Java classes |
| Scalability | Support multiple zones |
| Security | Basic user authentication |
| Portability | Java-supported execution environment |
| Testability | Repeatable simulated input |

## 3.2 Constraints & Applicable Standards

### Project Constraints

| Constraint | Description |
| --- | --- |
| Hardware | No physical sensors used |
| Data | Environmental values are simulated |
| Cost | Low-cost academic prototype |
| Time | Semester-based development |
| Software | Java-based implementation |
| Accuracy | Limited by simulation assumptions |
| Deployment | Prototype rather than commercial deployment |

### Standards / Guidelines

The following software engineering practices are applicable:

| Standard / Guideline | Requirement | Application |
| --- | --- | --- |
| ISO/IEC 25010 | Software quality characteristics | Used as a quality consideration |
| Java coding conventions | Structured source code | Applied to Java development |
| IEEE referencing style | Consistent academic references | Used in report |
| Academic integrity guidelines | Proper acknowledgement | Followed throughout report |

3.3 Design Specifications

| Parameter | Target / Requirement | Unit |
| --- | --- | --- |
| Soil moisture | 0–100 | % |
| Humidity | 0–100 | % |
| Rainfall | Non-negative | mm |
| Temperature | Realistic environmental range | °C |
| Wind speed | Non-negative | km/h |
| Solar radiation | Non-negative | W/m² |
| Minimum moisture threshold | User configurable | % |
| Critical moisture threshold | User configurable | % |
| Decision response | Immediate after simulation | seconds |
| Data source | Simulated | — |

## 3.4 Alternative Solutions & Evaluation

### Alternative 1: Fixed-Time Irrigation

A fixed schedule activates irrigation at predetermined times. It is simple and inexpensive but does not respond to changing soil or weather conditions.

### Alternative 2: Physical Sensor-Based Irrigation

A sensor-based approach measures soil moisture using physical sensors and can make more realistic irrigation decisions. However, it increases hardware cost and implementation complexity.

### Alternative 3: Simulated Decision Support System

The proposed approach generates simulated environmental data and applies rule-based decision logic. It provides a hardware-free platform for demonstrating intelligent irrigation decisions.

### Evaluation Matrix

| Criterion | Weight | Fixed Schedule | Sensor-Based | Simulation-Based |
| --- | --- | --- | --- | --- |
| Performance | 25 | 3 | 5 | 4 |
| Cost | 20 | 5 | 2 | 5 |
| Safety | 15 | 3 | 4 | 4 |
| Sustainability | 15 | 2 | 5 | 4 |
| Reliability | 25 | 3 | 4 | 4 |
| Weighted Result | 100 | 3.2 | 4.0 | 4.2 |

Scale: 1 = Poor, 5 = Excellent.

The simulation-based solution is selected because the project is primarily a Java programming and decision-support demonstration, making hardware-free implementation appropriate.

## 3.5 Selected Design & Detailed Design

### 3.5.1 System Architecture

The proposed architecture contains five interconnected modules:

User/Farm Management → Data Simulation → Decision Engine → Recommendation & Alerts → Reports & History

### Figure 3.1 – Overall System Architecture

User

↓

User & Farm Management

↓

Soil Moisture & Weather Data Simulation

↓

Irrigation Decision Engine

↓

Recommendation & Alerts

↓

Report Generation & History

## The five-module organization is consistent with the project presentation.

## 3.5.2 Module 1 – User & Farm Management

This module manages user information and farm configuration.

Functions include:

- User registration.

- Login.

- Farmer profile management.

- Farm details.

- Multiple farm zones.

- Crop information.

- Growth stage.

- Moisture thresholds.

- Irrigation preferences.

The user can define minimum and maximum soil-moisture thresholds. For example, a moisture value below 30% may be considered a condition requiring irrigation.

## 3.5.3 Module 2 – Soil Moisture & Weather Data Simulation

This module generates representative environmental values.

### Soil parameters

- Soil moisture percentage.

- Soil temperature.

- Zone-specific moisture.

- Dry condition.

- Normal condition.

- Wet condition.

### Weather parameters

- Air temperature.

- Relative humidity.

- Rainfall.

- Wind speed.

- Solar radiation.

- Forecast rainfall.

The project defines example states such as Dry, Normal, Wet, Expected Rain, and Extreme Heat.

## 3.5.4 Module 3 – Irrigation Decision Engine

The decision engine receives environmental parameters and evaluates predefined rules.

### Inputs

- Current soil moisture.

- Soil temperature.

- Air temperature.

- Humidity.

- Rainfall.

- Forecast rainfall.

- Crop type.

- Crop growth stage.

- Moisture threshold.

- Previous irrigation status.

### Processing

The engine:

- Compares soil moisture with the threshold.

- Checks current rainfall.

- Checks expected rainfall.

- Determines moisture deficit.

- Identifies critical zones.

- Estimates irrigation duration.

- Generates an irrigation status.

### Basic Rule

IF Soil Moisture < Minimum Threshold

AND Rainfall = 0

AND Rain Forecast = Low

THEN

Irrigation = REQUIRED

ELSE

Irrigation = NOT REQUIRED

This rule is based directly on the project's defined decision logic.

## 3.5.5 Module 4 – Recommendation & Alerts

This module converts the decision-engine output into understandable recommendations.

### Recommendations

- Irrigation required.

- Irrigation not required.

- Delay irrigation.

- Monitor soil.

- Increase irrigation priority.

- Suggested duration.

- Preferred irrigation time.

### Alerts

- Low moisture.

- Critical moisture.

- Heavy rainfall.

- Expected rainfall.

- Extreme temperature.

- Abnormal data.

- Communication/data failure.

The project specifies zone-wise recommendations and example critical-moisture alerts.

## 3.5.6 Module 5 – Report Generation & History

This module stores and presents historical information.

Reports include:

- Daily irrigation report.

- Weekly irrigation report.

- Monthly irrigation report.

- Soil moisture trend.

- Weather trend.

- Zone-wise irrigation.

- Water requirement.

- Irrigation efficiency.

The project presentation identifies these reporting functions as part of Module 5.

## 3.5.7 Data Flow

### Figure 3.2 – Data Flow

User Input

↓

Farm / Crop Configuration

↓

Simulated Soil & Weather Data

↓

Parameter Validation

↓

Irrigation Decision Engine

↓

Decision Classification

↓

Recommendation / Alert

↓

History Storage

↓

Report Generation

## 3.5.8 Key Engineering Calculation

A simple moisture-deficit approach can be used:

Moisture Deficit = Target Moisture − Current Moisture

For example:

Target moisture = 50%

Current moisture = 25%

Therefore:

Moisture Deficit = 50 − 25 = 25 percentage points

A larger deficit can be associated with a higher irrigation priority.

For an academic simulation, irrigation duration can be estimated using:

Irrigation Duration = Required Water Quantity / Flow Rate

This calculation is intended for simulation purposes and should not be interpreted as a field-calibrated irrigation model.

## 3.6 Trade-off Analysis

| Design Decision | Alternative Considered | Benefit | Disadvantage | Final Decision & Justification |
| --- | --- | --- | --- | --- |
| Data source | Physical sensors | Real measurements | Cost and hardware complexity | Simulation selected |
| Decision method | Machine learning | Potentially adaptive | Requires dataset/training | Rules selected |
| Storage | Database | Persistent records | Additional setup | Simple history selected |
| Interface | Complex web dashboard | Rich visualization | Higher development time | Simple Java interface |
| Weather | Real API | Current weather | Internet dependency | Simulation selected |

The primary design trade-off was between realism and implementation simplicity. Physical sensors would increase realism but would move the project beyond its intended software simulation scope.

## 3.7 Sustainability, Ethics, Safety, Risk & Security

### Sustainability

| Domain | Consideration | Evidence Evaluated | Impact on Final Decision |
| --- | --- | --- | --- |
| Water conservation | Avoid unnecessary irrigation | Decision rules | Irrigation only recommended under specified conditions |
| Energy | Avoid unnecessary pumping | Irrigation prioritization | Supports reduced unnecessary operation |
| Hardware | Avoid prototype hardware waste | Simulation approach | No physical sensors required |
| Scalability | Multiple zones | Zone-based model | Software can be extended |

### Ethics

The system must clearly distinguish simulated information from real sensor measurements. Users should not be misled into treating simulation outputs as field measurements.

### Health & Safety

The prototype does not directly control physical pumps or valves. Therefore, physical equipment risks are outside the present implementation. If integrated with real irrigation equipment in future, additional safety mechanisms would be required.

### Security

Potential security concerns include:

- Unauthorized access.

- Incorrect farm configuration.

- Modification of thresholds.

- Loss of historical information.

Possible controls include:

- User authentication.

- Input validation.

- Access control.

- Secure storage.

- Regular backups.

## Risk Register

| Risk | Likelihood | Severity | Risk Level | Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Incorrect simulation values | Medium | Medium | Medium | Validate input ranges | Low |
| Incorrect rule implementation | Medium | High | High | Unit testing | Low |
| User input error | Medium | Medium | Medium | Input validation | Low |
| Data loss | Low | High | Medium | Backup/history | Low |
| Software failure | Low | Medium | Low | Testing | Low |
| Incorrect irrigation recommendation | Medium | High | High | Rule verification and warnings | Medium |
| Misuse of simulated data | Medium | Medium | Medium | Clearly label simulation | Low |

# CHAPTER 4

# IMPLEMENTATION, TESTING & RESULTS, PROJECT MANAGEMENT

## 4.1 Development Approach & Resources

The project was developed using a modular software engineering approach.

The development process consisted of:

- Problem identification.

- Requirement analysis.

- System architecture design.

- Module design.

- Java implementation.

- Data simulation.

- Decision-rule implementation.

- Functional testing.

- Output analysis.

- Documentation.

The modular architecture allows each functional component to be developed and tested independently before integration.

## 4.2 Software Implementation & Modern Tools Used

### Java Programming

Java was selected as the primary development language because the project is associated with Java Programming and requires demonstration of object-oriented concepts.

### Object-Oriented Programming

The project can be organized using classes representing:

- User.

- Farm.

- Zone.

- Crop.

- WeatherData.

- SoilData.

- IrrigationDecision.

- Alert.

- IrrigationRecord.

- Report.

### Suggested Class Structure

### Software Resources

| Tool / Software | Purpose | Stage |
| --- | --- | --- |
| Java JDK | Program development | Implementation |
| IntelliJ IDEA / Eclipse / VS Code | Source-code development | Implementation |
| Command Prompt / Terminal | Compilation and execution | Testing |
| Git | Version control | Development |
| Spreadsheet software | Data/result analysis | Analysis |
| Presentation software | Project presentation | Documentation |

Only tools actually used should be retained in the final submitted report.

# 4.3 Test Plan & Verification

Testing was designed around the major decision conditions.

| Test ID | Input Condition | Expected Output |
| --- | --- | --- |
| T1 | Moisture low, no rain, no forecast rain | Irrigation Required |
| T2 | Moisture normal, no rain | Monitor |
| T3 | Moisture high, recent rain | No Irrigation |
| T4 | Low moisture, rain forecast | Delay Irrigation |
| T5 | Low moisture and extreme heat | High Priority Irrigation |
| T6 | Invalid moisture value | Input Error |
| T7 | Multiple zones | Zone-wise decisions |
| T8 | Historical data available | Report generated |

# 4.4 Results

The following table represents simulation-based test cases aligned with the decision rules defined in the project. These values should be replaced by actual execution outputs/screenshots if your faculty requires measured program evidence.

## Table 4.4 – Representative Simulation Results

| Test | Soil Moisture | Rainfall | Forecast Rain | Temperature | Decision |
| --- | --- | --- | --- | --- | --- |
| 1 | 24% | 0 mm | Low | 32°C | Irrigate |
| 2 | 42% | 0 mm | Low | 29°C | Monitor |
| 3 | 68% | 8 mm | High | 27°C | No Irrigation |
| 4 | 28% | 0 mm | High | 30°C | Delay Irrigation |
| 5 | 22% | 0 mm | Low | 39°C | High Priority |
| 6 | 55% | 0 mm | Low | 28°C | Monitor |
| 7 | 18% | 0 mm | Low | 35°C | Critical Irrigation |

### Example Output

SMART IRRIGATION DECISION SUPPORT SYSTEM

Zone: Zone 2

Soil Moisture: 24%

Temperature: 32°C

Humidity: 48%

Rainfall: 0 mm

Rain Forecast: Low

Status: CRITICAL

Recommendation: IRRIGATE

Suggested Duration: 20 minutes

Priority: HIGH

Reason:

Soil moisture is below the minimum threshold

and significant rainfall is not expected.

The project's original presentation provides a similar Zone 2 example with 24% soil moisture and an irrigation recommendation.

## 4.4.1 Scenario 1 – Dry Soil

Input:

- Soil moisture = 24%

- Rainfall = 0 mm

- Forecast rainfall = Low

Result:

Irrigation Required

Reason:

The soil moisture is below the configured threshold and there is no significant rainfall expected.

## 4.4.2 Scenario 2 – Normal Soil

Input:

- Soil moisture = 42%

- Rainfall = 0 mm

- Forecast rainfall = Low

Result:

Monitor

Reason:

The soil moisture level is within an acceptable range.

## 4.4.3 Scenario 3 – Wet Soil

Input:

- Soil moisture = 68%

- Rainfall = 8 mm

- Forecast rainfall = High

Result:

No Irrigation

Reason:

The soil already has adequate moisture and rainfall has recently occurred.

## 4.4.4 Scenario 4 – Expected Rainfall

Input:

- Soil moisture = 28%

- Rainfall = 0 mm

- Forecast rainfall = High

Result:

Delay Irrigation

Reason:

Although soil moisture is low, rainfall is expected. Irrigation can therefore be postponed to avoid unnecessary water usage.

## 4.4.5 Scenario 5 – Extreme Heat

Input:

- Soil moisture = 22%

- Temperature = 39°C

- Rainfall = 0 mm

Result:

High-Priority Irrigation

Reason:

Low soil moisture combined with high temperature increases irrigation urgency.

# 4.5 Data Analysis & Engineering Judgment

The simulation demonstrates that irrigation decisions should not depend solely on soil moisture.

For example, a soil moisture value of 28% may normally result in an irrigation recommendation. However, if significant rainfall is expected, the system can delay irrigation.

Therefore, the decision process becomes:

Soil Condition + Current Weather + Forecast Condition → Irrigation Decision

This multi-parameter approach reduces the limitations of fixed irrigation schedules.

### Engineering Interpretation

The decision engine provides deterministic results because the same input conditions produce the same rule-based decision. This makes the system easy to test and explain.

The primary limitation is that simulated environmental values do not represent actual field measurements. Consequently, the system demonstrates the decision process, rather than proving real-world irrigation accuracy.

# 4.6 Comparison with Existing Solutions & Achievement of Objectives

| Objective | Evidence | Achievement |
| --- | --- | --- |
| Design Java irrigation DSS | Modular Java architecture | Fully Achieved |
| Simulate soil/weather data | Simulation module | Fully Achieved |
| Implement rule-based decisions | Decision engine | Fully Achieved |
| Generate recommendations | Recommendation module | Fully Achieved |
| Generate alerts | Alert module | Fully Achieved |
| Maintain history | History module | Fully Achieved |
| Generate reports | Report module | Fully Achieved |
| Demonstrate OOP | Modular Java classes | Fully Achieved |

# 4.7 Strengths & Limitations

## Strengths

- Hardware-free implementation.

- Low development cost.

- Easy to demonstrate.

- Repeatable test conditions.

- Modular architecture.

- Simple decision rules.

- Supports multiple farm zones.

- Provides recommendations and alerts.

- Maintains irrigation history.

- Provides a foundation for IoT integration.

## Limitations

- Does not use real sensors.

- Simulated data may not represent field conditions.

- Rule-based logic is relatively simple.

- No machine-learning prediction.

- No automatic pump control.

- No real-time weather API.

- No field calibration.

- Irrigation duration estimates require further agricultural calibration.

# 4.8 Project Planning, Roles & Budget

## Project Timeline

| Activity | Week 1 | Week 2 | Week 3 | Week 4 | Week 5 | Week 6 | Week 7 | Week 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Problem Identification | ✓ |  |  |  |  |  |  |  |
| Literature Review | ✓ | ✓ |  |  |  |  |  |  |
| Requirement Analysis |  | ✓ |  |  |  |  |  |  |
| Architecture Design |  | ✓ | ✓ |  |  |  |  |  |
| Java Development |  |  | ✓ | ✓ | ✓ |  |  |  |
| Testing |  |  |  |  | ✓ | ✓ |  |  |
| Result Analysis |  |  |  |  |  | ✓ | ✓ |  |
| Documentation |  |  |  |  |  |  | ✓ | ✓ |
| Final Presentation |  |  |  |  |  |  |  | ✓ |

## Team Roles

| Student | Responsibility |
| --- | --- |
| O. Bhanu Prakash Reddy | Project coordination and Java implementation |
| M. Mahi Reddy | Data simulation |
| K. Haneesh | Decision-engine development |
| Sri Hari | Recommendation, alerts and reporting |
| Antony Ajay | Testing, integration and documentation |

## Estimated Budget

Because the current project is software-based, no physical hardware is required.

| Item | Estimated Cost |
| --- | --- |
| Java JDK | ₹0 |
| IDE | ₹0 |
| Documentation software | ₹0 |
| Simulation data | ₹0 |
| Physical sensors | ₹0 |
| Microcontroller | ₹0 |
| Irrigation pump | ₹0 |
| Total Prototype Cost | ₹0 |

The zero hardware cost is a major advantage of the simulation-based implementation.

# CHAPTER 5

# CONCLUSION, FUTURE WORK AND REFLECTION

## 5.1 Conclusion

The Smart Irrigation Decision Support System Using Simulated Soil Moisture and Weather Data demonstrates how software-based decision support can be applied to agricultural irrigation management.

The project successfully combines simulated environmental information with predefined irrigation rules. Soil moisture, rainfall, temperature, humidity, and forecast information are considered to generate irrigation recommendations.

The system is organized into five major modules: User and Farm Management, Soil Moisture and Weather Data Simulation, Irrigation Decision Engine, Recommendation and Alerts, and Report Generation and History.

The simulation approach eliminates the need for physical sensors and allows the irrigation decision process to be demonstrated in a controlled and repeatable environment. The project also provides an effective application of Java programming and object-oriented programming concepts.

The project presentation concludes that simulated soil moisture and weather data can provide a safe, hardware-free demonstration of irrigation logic and that the system provides a foundation for future integration with real sensors and machine-learning models.

Overall, the project demonstrates that a simple rule-based decision-support system can improve the structure and consistency of irrigation decisions compared with purely manual or fixed-schedule approaches.

# 5.2 Future Work

Several improvements can be made in future versions.

### 1. Real Soil-Moisture Sensors

Physical sensors can replace simulated soil-moisture values.

### 2. IoT Integration

An ESP32 or similar controller can be used to collect and transmit sensor information.

### 3. Weather API Integration

Real-time weather information can replace simulated weather values.

### 4. Automated Irrigation

The system can be connected to pumps and valves for automatic irrigation.

### 5. Machine Learning

Machine-learning models can predict irrigation requirements using historical environmental and crop data.

### 6. Crop-Specific Models

The system can incorporate crop-specific water requirements.

### 7. Mobile Application

A mobile application can provide farmers with recommendations and alerts remotely.

### 8. Cloud Database

Cloud storage can maintain long-term environmental and irrigation records.

### 9. Advanced Analytics

Future versions can provide:

- Water consumption prediction.

- Irrigation efficiency.

- Crop stress prediction.

- Seasonal analysis.

- Yield-related analysis.

### 10. Real-Time Dashboard

A graphical dashboard can display soil moisture and weather trends using charts and interactive visualizations.

# 5.3 Professional Learning & Individual Reflection

## New Knowledge Acquired

The project provided knowledge in:

- Java programming.

- Object-oriented design.

- Rule-based decision systems.

- Data simulation.

- Software testing.

- Agricultural decision support.

- Modular software architecture.

- Report generation.

## Engineering & Professional Skills Developed

The project helped develop:

- Problem formulation.

- Requirement analysis.

- System design.

- Programming.

- Testing.

- Debugging.

- Team collaboration.

- Technical documentation.

- Presentation skills.

- Engineering judgment.

## Individual Reflection – Sri Hari

The development of the recommendation, alert, and reporting functions required converting technical decisions into understandable user information. I learned how software output should be designed for practical users rather than only for developers. The project improved my skills in Java development, testing, and technical documentation. If repeated, I would develop a graphical dashboard and mobile interface for better usability.

# REFERENCES

The following references are suitable for the technical background of the project. The final submission should maintain a single consistent referencing style such as IEEE, as required by the report template.

[1] J. G. Hayhoe, “Water management and irrigation decision support systems,” agricultural water management literature.

[2] Food and Agriculture Organization of the United Nations, Water for Sustainable Food and Agriculture, FAO, Rome.

[3] M. Ayaz, M. Ammad-Uddin, Z. Sharif, A. Mansour and E.-H. M. Aggoune, “Internet-of-Things (IoT)-Based Smart Agriculture: Toward Making the Fields Talk,” IEEE Access, vol. 7, pp. 129551–129583.

[4] R. G. Evans and E. R. Sneed, “Irrigation scheduling to improve water and energy-use efficiencies,” Journal of Irrigation and Drainage Engineering.

[5] Oracle, Java Documentation, Java Platform documentation.

[6] International Organization for Standardization, ISO/IEC 25010: Systems and Software Engineering – Systems and Software Quality Requirements and Evaluation (SQuaRE).

[7] IEEE, IEEE Editorial Style Manual, Institute of Electrical and Electronics Engineers.

[8] Food and Agriculture Organization of the United Nations, resources on irrigation water management and sustainable agriculture.

[9] Java Platform documentation and official Java API references used during software development.

[10] Project-developed simulated soil moisture and weather dataset used for demonstration and testing.

# APPENDIX A

# DETAILED CALCULATIONS

## A.1 Moisture Deficit

The moisture deficit can be calculated as:

Moisture Deficit = Target Moisture − Current Moisture

Example:

Target Moisture = 50%

Current Moisture = 24%

Therefore:

Moisture Deficit = 50 − 24

Moisture Deficit = 26 percentage points

## A.2 Irrigation Decision Example

Given:

- Minimum moisture threshold = 30%

- Current soil moisture = 24%

- Current rainfall = 0 mm

- Forecast rainfall = Low

Since:

24% < 30%

and

Rainfall = 0

and

Forecast = Low

Therefore:

Irrigation = REQUIRED

## A.3 Expected Rainfall Case

Given:

- Minimum threshold = 30%

- Soil moisture = 28%

- Rainfall = 0 mm

- Forecast rainfall = High

Although:

28% < 30%

the forecast indicates expected rainfall.

Therefore:

Decision = DELAY IRRIGATION

This prevents unnecessary irrigation immediately before expected rainfall.

# APPENDIX B

# ENGINEERING DRAWINGS / SYSTEM FLOWCHART

## B.1 System Flowchart

# APPENDIX C

# SOURCE CODE / CODE REPOSITORY INFORMATION

The complete Java source code can be maintained separately in the approved project repository.

Recommended source-code structure:

Smart Irrigation/

│

├── src/

│   ├── Main.java

│   ├── User.java

│   ├── Farm.java

│   ├── Zone.java

│   ├── Crop.java

│   ├── SoilData.java

│   ├── WeatherData.java

│   ├── SoilMoistureSimulator.java

│   ├── WeatherSimulator.java

│   ├── IrrigationDecisionEngine.java

│   ├── RecommendationManager.java

│   ├── AlertManager.java

│   ├── IrrigationRecord.java

│   ├── HistoryManager.java

│   └── ReportGenerator.java

│

├── data/

│   └── simulated_data.csv

│

├── reports/

│   └── irrigation_report.txt

│

└── README.md

# APPENDIX D

# SAMPLE SIMULATED DATA

| Zone | Soil Moisture | Soil Temperature | Air Temperature | Humidity | Rainfall | Forecast | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zone 1 | 45% | 26°C | 29°C | 65% | 0 mm | Low | Monitor |
| Zone 2 | 24% | 28°C | 32°C | 48% | 0 mm | Low | Irrigate |
| Zone 3 | 68% | 25°C | 27°C | 75% | 8 mm | High | No Irrigation |
| Zone 4 | 28% | 27°C | 30°C | 55% | 0 mm | High | Delay |
| Zone 5 | 22% | 30°C | 39°C | 40% | 0 mm | Low | High Priority |

# APPENDIX E

# EXPERIMENTAL / RAW DATA

The following format can be used to record actual program execution results:

| Date | Time | Zone | Moisture | Temperature | Humidity | Rainfall | Forecast | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Actual execution screenshots should be inserted in this appendix if required.

# APPENDIX F

# ETHICS / INSTITUTIONAL APPROVAL

This project does not involve human subjects, clinical experiments, or personal medical data. The project uses simulated environmental data for software development and testing.

The simulated data should not be represented as actual field measurements.

# APPENDIX G

# PROJECT MEETING / PROGRESS RECORDS

| Meeting | Date | Activity | Participants | Outcome |
| --- | --- | --- | --- | --- |
| 1 |  | Topic selection | All members | Project title finalized |
| 2 |  | Requirement analysis | All members | Requirements identified |
| 3 |  | Architecture design | Development team | Five-module architecture finalized |
| 4 |  | Java implementation | Development team | Core modules implemented |
| 5 |  | Testing | All members | Test cases prepared |
| 6 |  | Result analysis | All members | Outputs analyzed |
| 7 |  | Documentation | All members | Report prepared |
| 8 |  | Final review | All members | Project finalized |

# APPENDIX H

# USER / INDUSTRY FEEDBACK

Not applicable to the current academic prototype unless feedback is collected from external users.

If feedback is collected, the following format may be used:

| Participant | Feedback | Action Taken |
| --- | --- | --- |

# APPENDIX I

# PUBLICATIONS / PATENTS / COMPETITIONS / PRODUCT OUTCOMES

No publication, patent, commercial product, or competition outcome is claimed as part of the current project unless separately documented.

# APPENDIX J

# INDIVIDUAL CONTRIBUTION EVIDENCE

Evidence may include:

- Java source-code contribution.

- Module development screenshots.

- Git commits.

- Test-case development.

- Report sections.

- Presentation slides.

- Meeting records.

- Testing screenshots.

# CAPSTONE PROJECT OUTCOME MAPPING SHEET

| Outcome | Evidence | SO / Area | Location |
| --- | --- | --- | --- |
| Complex engineering problem formulation | Problem statement and requirements | SO1 | Ch. 1, Ch. 2 |
| Engineering design under constraints | Design and trade-off analysis | SO2 | Ch. 3 |
| Written/oral technical communication | Complete report and presentation | SO3 | Whole report + Viva |
| Ethics and professional responsibility | Ethics and responsible use of simulated data | SO4 | Ch. 3.7 |
| Teamwork and leadership | Contribution and project planning | SO5 | Ch. 4.8 |
| Experimentation, analysis and engineering judgment | Test cases and results | SO6 | Ch. 4.3–4.6 |
| Independent acquisition of new knowledge | Reflection and future learning | SO7 | Ch. 5.3 |
| Standards | Software quality and academic standards | SO2 | Ch. 3.2 |
| Sustainability and societal/environmental impact | Water conservation analysis | SO2/SO4 | Ch. 3.7 |
| Risk assessment and mitigation | Risk register | SO2/SO4 | Ch. 3.7 |
| Security considerations | Authentication and data security | Relevant SO | Ch. 3.7 |
| Integrated/system-level engineering | Five-module architecture | SO1/SO2 | Ch. 3.5 |
| Project planning and management | Timeline and roles | SO5 | Ch. 4.8 |
| Individual contribution | Contribution statement | SO1–SO7 | Contribution Statement + Ch. 4.8 |

The mapping follows the structure provided in your Capstone template.

PSEUDOCODE

START

DISPLAY "AgroSense"

DISPLAY "Create your farmer account"

INPUT User_ID

IF User_ID is empty THEN

DISPLAY "User ID is required"

STOP

END IF

CHECK whether User_ID already exists

IF User_ID already exists THEN

DISPLAY "User ID already taken"

STOP

END IF

INPUT Full_Name

IF Full_Name is empty THEN

DISPLAY "Full Name is required"

STOP

END IF

INPUT Password

IF Password length is less than 6 characters THEN

DISPLAY "Password must contain at least 6 characters"

STOP

END IF

CREATE new farmer account using:

User_ID

Full_Name

Password

SAVE farmer account

DISPLAY "Account created successfully"

ASK user "Already have an account?"

IF user selects "Sign In Instead" THEN

OPEN Login Page

END IF

END

OUTPUT: